|  | Simulation of Chrankshaft Date: Wednesday, September 18, 2019 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 4 Mesh information 5 Study Results 7 Conclusion 10. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Chrankshaft Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror2; Solid Body; Mass:1.93114 lb Volume:6.94204 in^3 Density:0.27818 lb/in^3 Weight:1.92983 lbf; E:\Week 4B\Chrankshaft.SLDPRT Sep 18 12:54:46 2019.

# Material Properties Model Reference; Properties; Components. Name:; Alloy Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 89,984.6 psi. Tensile strength:; 104,982 psi.; SolidBody 1(Mirror2)(Crank Shaft).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Reference:; Top. Type:; Apply force. Values:; ---, ---, 1,000 lbf..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; On. Jacobian points; 4 Points. Element Size; 0.19082 in. Tolerance; 0.0095411 in. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 13326. Total Elements; 7837. Maximum Aspect Ratio; 6.1775. % of elements with Aspect Ratio < 3; 97.9. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 9.624e-01 psi Node: 12511; 2.890e+04 psi Node: 1349. Chrankshaft-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0.000e+00 in Node: 121; 5.240e-03 in Node: 10681. Chrankshaft-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed shape. Chrankshaft-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 3.114e+00 Node: 1349; 9.350e+04 Node: 12511. Chrankshaft-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion