|  | Simulation of PipeReducer Date: Wednesday, September 18, 2019 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 4 Mesh information 6 Study Results 8 Conclusion 11. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: PipeReducer Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. CSK for 5/8 Flat Head Machine Screw (100)1; Solid Body; Mass:48.9034 lb Volume:152.079 in^3 Density:0.321566 lb/in^3 Weight:48.8702 lbf; E:\Week 4B\Pipe Reducer\PipeReducer.SLDPRT Oct 9 08:18:10 2015.

# Material Properties Model Reference; Properties; Components. Name:; Copper. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 37,513.4 psi. Tensile strength:; 57,200 psi.; SolidBody 1(CSK for 5/8 Flat Head Machine Screw (100)1)(REDUCER-MS).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 5 face(s). Type:; Fixed Geometry.. Fixed-2; Entities:; 13 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Pressure-1; Entities:; 3 face(s). Type:; Normal to selected face. Value:; 1,500. Units:; psi..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.533579 in. Tolerance; 0.0266789 in. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 19418. Total Elements; 10327. Maximum Aspect Ratio; 6.8623. % of elements with Aspect Ratio < 3; 96.9. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 3.111e-01 psi Node: 5284; 1.507e+04 psi Node: 1943. PipeReducer-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0.000e+00 in Node: 517; 2.776e-03 in Node: 1357. PipeReducer-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed shape. PipeReducer-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 2.489e+00 Node: 1943; 1.206e+05 Node: 5284. PipeReducer-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion